# Karma-Based Credit Scheduler (KBCS) Implementation Plan

## 1. Objective

To implement a Karma-Based Credit Scheduler (KBCS) on a BMv2 P4 switch to improve fairness between aggressive (e.g., CUBIC) and fragile (e.g., BBR) TCP flows at a bottleneck link.

## 2. System Architecture

• Data Plane: BMv2 Simple_Switch running P4 program.

• Control Plane: Mininet topology with 1 switch and 3 hosts.

• Bottleneck link configured at 10 Mbps.

• Traffic generated using iperf3.

## 3. Network Topology

• h1: Aggressive flow (TCP CUBIC).

• h2: Fragile flow (TCP BBR).

• h3: Receiver.

• s1: BMv2 P4 switch.

• Bottleneck between switch and h3 (10 Mbps).

## 4. Data Plane Implementation (P4)

### 4.1 headers.p4

• Ethernet header definition.

• IPv4 header definition.

• TCP header definition.

• Custom metadata structure including:

- karma_score (8 bits)

- queue_id (2 bits)

### 4.2 kbcs.p4

Parser: Extract Ethernet, IPv4, and TCP headers.

Ingress Pipeline:

• Perform L3 forwarding.

• Identify flow using 5-tuple (srcIP, dstIP, srcPort, dstPort, protocol).

• Maintain per-flow byte counters using registers.

• Compute Karma score based on rate threshold.

• Map flow to queue based on Karma score:

- High Karma → Gold Queue (0)

- Medium Karma → Silver Queue (1)

- Low Karma → Bronze Queue (2)

Deparser: Reconstruct packet headers.

## 5. Traffic Manager Configuration

Configure BMv2 queues with Weighted Round Robin scheduling:

• Queue 0 (Gold): Weight 4

• Queue 1 (Silver): Weight 2

• Queue 2 (Bronze): Weight 1

## 6. Control Plane Implementation

• Define topology.json for Mininet.

• Use TCLink to configure 10 Mbps bottleneck.

• Enable congestion control algorithms:

- h1: TCP CUBIC

- h2: TCP BBR

• Use Makefile to compile P4 and launch Mininet.

## 7. Verification Plan

### 7.1 Baseline Testing (Without KBCS)

• Run iperf3 traffic from h1 and h2 to h3.

• Measure throughput, RTT, and fairness index.

• Observe dominance of aggressive flow.

### 7.2 KBCS Enabled Testing

• Enable KBCS logic in P4.

• Repeat traffic experiments.

• Measure throughput and RTT.

• Calculate Jain’s Fairness Index.

## 8. Fairness Evaluation

Jain’s Fairness Index formula:

J = (x1 + x2)^2 / (2 * (x1^2 + x2^2))

Where x1 and x2 are throughputs of the two flows.

## 9. Expected Outcome

• Aggressive flows receive lower priority queues.

• Fragile flows receive higher priority queues.

• Improved fairness at bottleneck link.

• Higher Jain’s fairness index compared to baseline.